AMD

SSE, PCLAN, NPD

Test and Qualification Plan

for

Magic Packet Generation Utility

Version 1.0

July 31, 1997

____________________________________
Test and Support Manager				Software Development Manager

____________________________________
Hardware Development Manager			Marketing Manager

Table of Contents

1.1 Project Outline	3

1.2 Intended Audience	3

1.3 Purpose of the Document	3

2. Test Strategy  and Schedule	          3

2.1 Test Objectives	3

2.2 Test Schedule	3

3. Hardware Software Platforms	3

3.1 Operating Systems.................................................................................................................................4

3.2 Hardware Platforms............................................................................................................................4

3.3 Ethernet Software Drivers..........................................................................................................................4

3.4 Network Interface Cards	4

4. Test Procedures	5

4.1 Power On One Host	5

4.2 Group Power On

4.2.1 Day and time Variations	6

4.2.2 All Days Included...............................................................................................................................6

5. Test Reports	6

# Introduction

## Project Outline

This utility will be developed as a 32-bit Windows application for Windows 95 and NT. It uses the Windows socket API v1.1 to interface with TCP/IP protocol stack in order to generate a Magic Packet

conformant Ethernet frame using IP broadcast addressing. This technique will improve the propogation

of Magic Packets through routed networks. Note that the TCP/IP requirement is for the mangement console

only and does not prelude the other network protocols across the LAN/WAN. However, it is anticipated that

some advanced features, like auto-discovery of nodes, will only be available on a distributed TCP/IP network.

## Intended Audience

This document is intended to be used by project managers and test engineers to define the scope of testing

activities to be performed during the testing cycle of this project.

## Purpose of The Document

This test plan defines the test requirements and test strategy necessary to ensure the following :

- All test objectives listed in section 4.0 are met

- Test schedule identified in section 2.2 is met

- Allow upper management to gauge the scope of the test effort and test coverage

- Allow FAE’s to communicate with OEM customers about the scope of the test effort

- Provide guidelines to the test and qualification team during the test phases

- Facilitate comprehensive test reporting and understanding of test reports

# Test Strategy  and Schedule

## Test Objectives

- Compatibility testing with applicable operating systems

- To ensure that features of the program function as expected

## Test Schedule

The following is the test schedule, based on the test phases.

| Test Phase | Start | End |
| --- | --- | --- |
| Test Functionality of Magic Packet Utility | 8/15/97 | 8/22/97 |
| Documentation | 7/31/97 | 8/29/97 |
| Final Release for packaging | 9/7/97 | 9/9/97 |

# Hardware Software Platforms

This section lists the hardware and software platforms that will be used during the testing of this release.  Details on the configurations of hardware and software are available on a test case basis.  Test cases to be used during the testing cycle are documented in Section ??.

## Operating Systems

Network Operating Systems

Microsoft Windows NT (v. 4.0)

Microsoft Windows 95 (v. 950 and OSR2)

## Hardware Platforms

The following hardware platforms will be used for testing the Magic Packet Utility.

| System name/model | CPU/Speed | Server/Client | Chipset, BIOS |
| --- | --- | --- | --- |
| Gateway 2000 | Pentium/166 MHZ | Server* | Intel |
| HP Vectra | Pentium/100 MHZ | Client* | VLSI |

*Note:  The Server is the system which has the Magic Packet Utility installed. The client(s) have Windows 95 or Windows NT 4.0 installed and are awaken by a Magic Packet sent by the Server.

## Ethernet Software Drivers Used

NDIS3 MAC Driver

NDIS4 Miniport Driver (Driver is valid for Window 95 OSR2 and Windows NT 4.0)

## Network Interface Cards

The following LT NICs will be used for testing the v3.3 drivers and installation program:

| LT Card Name / Type | Specific Settings | Speed (Mbps) |
| --- | --- | --- |
| PCnet-Fast (Installed on Server) | None | 10 |
| PCnet-PCI II (Magic Packet Enabled) | None | 10 |

*  Pcnet-PCI II was also tested on the Server to ensure compatibility.

# Test Procedures

This section contains detailed test procedures that will be used during the  test phase.

## Power on One Host

Objective:            This test wakes up one host via Magic Packet which shows the driver can be

loaded/unloaded dynamically under Windows 95 and Windows NT 4.0.

Steps :		1.    On a client system whose BIOS supports Power Management, install Win95.

- Load the network device driver by diskette.

- Reboot the system.

- Make sure the Power Management feature is set within Win95.

- Click on “Start” and “Shutdown” to put the client to sleep.

- Start the Magic Packet Utility program on the server system.

- Select Magic Packets and click on Power On One Host.

- In the section asking for the Broadcast IP Address type n 255.255.255.255 and for the Destination address type in the Adapter address.

- Wait for several seconds then click on the Send button in the Magic Packet program which is installed on the Server.

Pass Criteria : 	The client system should wake-up upon sending the Magic Packet.

## Group Power On

Objective:	Verify that the program can wake up a group of the systems on the network by

sending a Magic Packet to the sytems on the network.

Procedure:

- On two client systems whose BIOS supports Power Management, install Win95.

- Load the network device driver by diskette.

- Reboot the system.

- Click “Start” and “Shutdown”.

- Start the Magic Packet Utility program on the server system.

- Select Magic Packets and click on Create a List of Hosts .

- After list has been created click on upper left corner to close the box which shows the systems on the network.

- Select Magic Packets and click on Group Power On.

- Select the name of the file which you created or the default file which is Newgroup.mp. Make sure that the IP Broadcast address of 255.255.255.255 is set.

- Select the appropriate day and time and click Add then click OK.

- The utility sends out a Magic Packet on the network at the time specified which in turn wakes up the two HP systems on the network.

### Day and Time Variations

- On the Server select Magic Packets and click on Group Power On.

- Click Browse and select the file which was created when a list of hosts was created (NewGroup.mp is the default filename).

- Check Monday and set time for 2:30 p.m..

- Click on Add and then OK.

- A Magic Packet will be sent at 2:30 to wake up all sleeping clients which exist in the NewGroup.mp file.

- Repeated procedure for the different days of the week.

### All Days Included

- On the Server select Magic Packets and click on Group Power On.

- Click Browse and select the file which was created when a list of hosts was created (NewGroup.mp is the default filename).

- Check all seven days of the week and set the time for 9:00 a.m..

- Click Add and then OK.

- A Magic Packet will be sent at 9:00 a.m. every morning to wake up all sleeping clients which exist in the NewGroup.mp file.

# Test Report

## Power on One Host

Objective: 	This test wakes up one host via Magic Packet which shows the driver can be

loaded/unloaded dynamically under Windows 95 and Windows NT 4.0.

Test Results: Pass

### Group Power On

Objective:	Verify that the program can wake up a group of the systems on the network by

sending a Magic Packet to the sytems on the network.

Test Results: Pass

5.2.1 Day and Time Variations

Objective:	To send a Magic Packet to the sleeping client system on one day of the week at any given

time.

Test Results:

Monday:      Pass

Tuesday:      Pass

Wednesday: Pass

Thursday:    Pass

Friday:       Pass

Saturday:   Pass

Sunday:     Pass

5.2.2 All Days Included

Objective:	To send a Magic Packet to the sleeping client systems on all days of the week at the same

time of day.

Test Results: Pass